ez.exe is the main control program you`ll use to hack around with your psx.

Here is the list of commands...

run

runbin

load

save

exec

jump

fill

sw

sb

resume

status		- not exactly very useful for you lot...

regs

setreg

bp		- not fully supported in this release

boot

cheat		- not fully supported in this release

nocheats	- not fully supported in this release

sv

lv

flush

console

flash

Here are the DOCS:

-------------------------------------------------------------------------------------------------

to run a PSX EXE file just type:-

ez run exefile.exe

-------------------------------------------------------------------------------------------------

to run a binray file just type:-

ez runbin binfile.bin

-------------------------------------------------------------------------------------------------

to send a file to the PSX just type:- (obviously you can change the address)

ez load file.bin 80010000

-------------------------------------------------------------------------------------------------

to grab some ram from the psx just type:- (obviously you can change the addresses)

ez save data.bin 80010000 80020000

-------------------------------------------------------------------------------------------------

to jump to a ram location on the psx just type:-

ez exec bfc00000

OR

ez jump bfc00000

-------------------------------------------------------------------------------------------------

to fill the psx ram with some value just type:-

ez fill 80010000 80020000 ff

-------------------------------------------------------------------------------------------------

to patch a longword of playstation ram just type:-

ez sw 80010000 ff00ff00

-------------------------------------------------------------------------------------------------

to patch a byte of playstation ram just type:-

ez sb 80010000 ff

-------------------------------------------------------------------------------------------------

to resume execution of the playstation (after a download or any command) just type:=

ez resume

-------------------------------------------------------------------------------------------------

to have a look at the current state of the registers on the psx just type:-

ez regs

-------------------------------------------------------------------------------------------------

to reset your psx just type:-

ez boot

-------------------------------------------------------------------------------------------------

to suck the vram of the psx just type:-

ez sv file.raw -raw

OR

ez sv file.tga -tga

-------------------------------------------------------------------------------------------------

to upload new data to vram on the psx just type:-

ez lv file.raw -raw

-------------------------------------------------------------------------------------------------

If your comms link is fucked up just type:-

ez flush

-------------------------------------------------------------------------------------------------

if you want to load/save files and see the printfs just type:-

ez console

-------------------------------------------------------------------------------------------------

if you want to reflash your action replay with a new rom just type:-

to flash it with the ezoray rom:

ez flash

or to flash it with someone elses rom (ie caetla)

ez flash caetla.rom

-------------------------------------------------------------------------------------------------

Please note:

you must have the enviroment variable AR_PORT setup to the base address of your datel comms port

just type:-

set AR_PORT=3xx

-------------------------------------------------------------------------------------------------

Obviously there is a ton of other stuff you can do, try typing:-

ez

-------------------------------------------------------------------------------------------------

EZORAY TAKE NO RESPONSIBITLY FOR YOU FUCKING YOUR ACTION REPLAY WITH THIS ROM, IF YOU DO, THATS YOUR SHITTY LUCK MATE

if you enjoy this rom, remember to greet SNAKE AND MCBAIN OF EZ-O-RAY

thats all for now... Snake / Mcbain 17/10/97

Sorry for the scrappy docs... but hey, at least we bothered to type a doc...